### Project Title

Design of an Isolated Power Supply for Cell Phone Charging Application

### Project Statement

The objective is to design an isolated power supply unit (PSU) capable of producing a regulated 5V DC output from a universal single-phase AC power outlet (85V to 265V RMS, 60Hz) for cellphone charging. The PSU is rated for a maximum power output of 30W and requires galvanic isolation between the input and output to ensure safe and reliable operation across varying input voltage levels.

### Project Progress Summary with Calculations

#### Step 1: Power Converter Topology Selection

- Chosen Topology: Flyback Converter

  - Justification: The flyback converter is well-suited for isolated, low-to-medium power applications (up to ~100W) and provides galvanic isolation through a transformer. It also supports wide input voltage ranges, which is essential for a universal AC input.

  - Challenges and Mitigations:

- High Voltage Stress: Flyback converters experience high voltage stresses on the MOSFET and transformer. We’ll incorporate a snubber circuit to handle transients and voltage spikes.

- Output Ripple: High output ripple is common in flyback designs, so we’ll select a low-ESR capacitor to minimize this.

  - Alternative Topologies: While a forward converter or push-pull converter could also be used, the flyback offers a simpler design and cost-effectiveness for a 30W application, thus making it a better choice for our goals.

#### Step 2: Design and Simulation Setup

##### Part A: Define Key Specifications

- Input Voltage (DC Equivalent) Calculation

  - The AC input voltage range of 85V to 265V RMS is converted to DC after rectification and filtering:

- Output Requirements:

  - Output Voltage Vout=5 V DC

  - Output Power Pout=30W

  - Output Current Calculation: Iout=Pout/ Vout= 30W / 5 V=6 A

- Switching Frequency Selection:

  - An initial switching frequency of 70 kHz was chosen to balance efficiency, transformer size, and control simplicity. Frequency adjustments will be considered within a range of 50-100 kHz if simulation results indicate improvements in performance.

- Duty Cycle Range:

  - The duty cycle will vary with input voltage to maintain a stable 5V output. Calculations and simulations will ensure it remains within practical limits (0.1 < D < 0.5).

##### Part B: Transformer Design - Turns Ratio Calculation

- Establish Turns Ratio Using the Flyback Converter Equation:

  - The flyback converter’s output voltage is

  - Rearranging for the turns ratio:

- Calculation of Turns Ratio for Minimum Input Voltage

  - Assumption for Duty Cycle: To achieve an approximate target output of 5V, we start by assuming an initial duty cycle of 0.4 for the minimum input voltage:

  - Initial Turns Ratio Selection: We’ll use a 16:1 primary-to-secondary turns ratio to ensure stability at both high and low input conditions.

- Verification of Turns Ratio at Maximum Input Voltage

  - With a 16:1 turns ratio, the duty cycle at maximum input (375V) is calculated as:

- Feedback Control: This indicates that the converter will operate within safe duty cycle limits (0.1 < D < 0.5) when regulated by a feedback-controlled PWM.

- Transformer Design Considerations:

  - Core Selection: A ferrite core is selected for its suitability in high-frequency applications, preventing core saturation at 70 kHz.

  - Winding Techniques: To minimize leakage inductance and voltage spikes, we’ll use optimized winding techniques.

##### Part C: MATLAB/Simulink Circuit Simulation Setup

- Simulation Circuit Components and Configuration

  - Input Stage: Use a DC voltage source to simulate the rectified AC input, ranging from 120V to 375V DC.

  - Primary Side:

    - Switching MOSFET: Control with a PWM signal to regulate duty cycle.

    - Flyback Transformer: Configured with the calculated 10:1 turns ratio.

    - Snubber Circuit: An RC snubber network protects the MOSFET from high voltage transients during switching.

  - Secondary Side:

    - Diode: For rectification.

    - Output Capacitor: Smooths output ripple to achieve steady 5V DC.

- Control Circuit Configuration

  - Feedback Loop: A feedback control mechanism will adjust the duty cycle of the PWM signal based on the output voltage to ensure regulation at 5V across all input conditions.

- Simulation Execution and Waveform Capture

  - Input Conditions: The simulation will be conducted at 120V DC (minimum) and 375V DC (maximum) to test the converter’s performance.

  - Waveforms Captured: Primary and secondary voltage/current waveforms, as well as output voltage and current, will be recorded for analysis.

### XXXX: Run LTSPICE Simulation

### XXXX: Component Selection

Voltage and Current Stress Calculations

- MOSFET: Calculate the maximum voltage and current it must handle based on simulation results.

- Diode: Ensure the selected diode can withstand the peak reverse voltage and forward current.

- Capacitors and Snubber Circuit: Select capacitors with suitable voltage and ripple current ratings, and confirm the snubber network can handle transient voltages.

Component Selection from Suppliers

- Use Digikey or Mouser to find specific components that meet the calculated requirements, and document part numbers, voltage/current ratings, and other specifications.

### XXXX:  Loss Calculation and Efficiency Estimation

Types of Losses to Calculate:

- Switching Losses: Due to turning the MOSFET on and off.

- Conduction Losses: From current flowing through the MOSFET and diode during operation.

- Core Losses in Transformer: Due to hysteresis and eddy currents in the transformer core.

- Copper Losses in Transformer Windings: Caused by resistance in the windings.

Efficiency Estimation:

- Sum the total losses to determine power lost in the circuit.

### XXXX: Final Documentation and Reporting

- Technical Report Compilation

  - Include each design step, including calculations, component selection, simulation results, and efficiency estimates.

  - Sections to Cover:

    - Introduction

    - Topology Selection and Justification

    - Circuit Diagram and Operation Principle

    - Simulation Results and Analysis

    - Component Selection

    - Loss Calculation and Efficiency Estimation

    - Conclusion